# DCMA NSEO MPR/MPS CHECKLIST #03

# Addendum 1 - NDT QUALIFICATION, CERTIFICATION AND OVERSIGHT

| SUPPLIER & CAGE: |  |
| --- | --- |
| LOCATION: |  |
| Program Type: Level I/SUSBAFE (LI/SS); Navy Propulsion Program (NPP); Deep Submergence Systems/Scope of Certification Program (DSS-SOC). Nuclear Plant Material (NPM); Naval Nuclear Propulsion Program (NNPP); Aircraft Launch & Recovery Equipment (ALRE). Fly By Wire Ships Control Systems (FBWSCS); Ships Critical Safety Items (SCSIs); Other:. Contractual Requirement(s) for this process: Supplier Procedure Number(s), Title(s) & Revision Level(s)/Date(s): Surveillance Performed By:. Date(s) of Surveillance:. Contract Number(s):. Part Number(s)/Serial number(s)/NSN:. Part Nomenclature(s):. Supplier Personnel Contacted and Titles:. Drawing Number & Revision:. |  |  |  |

# Process Concerns and Guidance:

- Written Practice: Navy NDT specifications require the use and adherence of an Examiner approved qualification and certification program procedure, i.e. Written Practice.

- Initial NDT Training: Initial training courses are an important aspect of the NDT program that imparts the basic minimum information related to a specific NDT discipline to the individual inspector candidate or trainee.

- Work Time Experience: T9074-AS-GIB-010/271 requires the accumulation of work time experience prior to attempting certification for the first time for each method.

- Certification Process: The certification process entails administering written and practical exams that meet specific criteria and requirements.

- Oversight: Navy specifications require administration of NDT Inspector Oversight in the form of surveillances, re-inspections and Technical Performance Evaluations.

- Examiner Responsibilities: All Navy NDT specifications contain numerous Examiner responsibilities that must be adhered to in the administration of the NDT program.

Governing Specifications:

- Recommended Practice No. SNT-TC-1A

- T9074-AS-GIB-010/271

- MIL-STD-2132

- NS250-1500-1

- NSTR-99

| No. | Checklist Item | Yes | No | N/A | Remarks / Method of Compliance |
| --- | --- | --- | --- | --- | --- |
| 1 | WRITTEN PRACTICE (TP271-1.6, TC1A- 5.0, NSTR99-2.0) |  |  |  |  |
| a | Is there a Level III Examiner approved NDT Written Practice? |  |  |  |  |
| b | Does the written practice contain, identify and meet the minimum requirements for the control and administration of the NDT program for each Level of Certification by method and technique for: |  |  |  |  |
|  | - Personnel training. |  |  |  |  |
|  | - Examinations. |  |  |  |  |
|  | - Certification/Recertification. |  |  |  |  |
|  | - Describe the responsibilities of each level of certification. |  |  |  |  |
|  | - Minimum training hours |  |  |  |  |
|  | - Minimum experience (WTE) |  |  |  |  |
|  | - Test techniques within each test method applicable to its scope of operations. |  |  |  |  |
| 2 | OVERSIGHT PROGRAM (TP271-1.6.10, TC1A-10.0, NSTR99-3.0) |  |  |  |  |
| a | Is there an NDT oversight and surveillance program or plan? |  |  |  |  |
| b | Is it specified in the written practice or other document? |  |  |  |  |
| c | Does it address the following items: |  |  |  |  |
|  | - Oversight of NDT personnel (excluding Examiners). |  |  |  |  |
|  | - Technical Performance Evaluations, by the end of the second year of certification.(TP271) |  |  |  |  |
|  | - Monitoring surveillances of each inspection method certified, performed annually. |  |  |  |  |
|  | - Reinspecting previously accepted product. |  |  |  |  |
|  | - Monitoring within 6 months of initial certification for each inspection method qualified and then annually thereafter. (NSTR-99) |  |  |  |  |
|  | - Reinspection of previously accepted product on each inspector in each method qualified once during certification cycle. (Not to be performed in conjunction with monitoring). |  |  |  |  |
| 3 | RECORDS (TP271, TC1A, NS250-1500, MS2132, NSTR99) |  |  |  |  |
| a | Is there an individual “NDT personnel qualification”, certification and monitoring record for each inspector? |  |  |  |  |
| b | Do the records contain the following information as a minimum? |  |  |  |  |
| c | T9074-AS-GIB-010/271 Rev. 1 (para. 1.6.8) and SNT-TC1A-2016 (para.9.4). |  |  |  |  |
|  | - Name of the certified Inspector. |  |  |  |  |
|  | - Level of certification, NDT method and/or technique. |  |  |  |  |
|  | - Educational background and experience including training prior to initial certification. |  |  |  |  |
|  | - Vision exam for current and preceding certification period, including corrective aids. |  |  |  |  |
|  | - Statement indicating satisfactory completion of training IAW the employer’s written practice with Level III Examiners signature. |  |  |  |  |
|  | - Current exam copies or evidence of successful completion of examinations that can be correlated to the exam administered. |  |  |  |  |
|  | - Results of exams with composite grades or suitable evidence of grades. |  |  |  |  |
|  | - Current ASNT certification letter for each method for basic and method exams for Examiners. |  |  |  |  |
|  | - Dates of certification and/or recertification. |  |  |  |  |
|  | - Certification expiration date. |  |  |  |  |
|  | - Certification statement with the Level III Examiners signature for Level I and II’s. Signature of the employer’s representative for Level III Examiners. |  |  |  |  |
| d | NSTR-99 para. 4.0 (NS250-1500-1, MS2132) |  |  |  |  |
|  | - Name of Employee and qualification level |  |  |  |  |
|  | - NDT qualification records, including the graded written and operational exams. |  |  |  |  |
|  | - Dates of monitoring including method and technique monitored |  |  |  |  |
|  | - Specific identification of product used for qualification and monitoring. |  |  |  |  |
|  | - Deficiencies noted, or other remarks as applicable. |  |  |  |  |
|  | - Corrective action if deviancies are noted. |  |  |  |  |
|  | - Records of reinspections |  |  |  |  |
| 4 | EXAMINATIONS (TP271-1.6.4, MS2132-10.5.2, NS1500- 6.7.7.1) |  |  |  |  |
| a | Are all exam materials or data bases adequately secured to prevent unauthorized disclosure or use? |  |  |  |  |
| b | Are level I and II personnel recertified to the same extent as the original certification every three years? |  |  |  |  |
| c | Are Examiners recertified to the same extent, (except the basic exam), as the original certification every 5 years? |  |  |  |  |
| d | T9074-AS-GIB-010/271 Rev. 1, SNT-TC1A-2016 |  |  |  |  |
|  | - Does the responsible Level III approve all written and practical qualification exams? (TC1A 8.1.1) |  |  |  |  |
|  | - Are there general (theory) written exams for Level I and II with at least 40 questions for each method? (TC1A-Table 8.3.4) |  |  |  |  |
|  | - Are the general written exam questions verbatim from the “ASNT Questions & Answers Books, (this is not allowed)? (TC1A- 8.6) |  |  |  |  |
|  | - Are there specific (specifications) written exams for Level I and II with at least 20 questions for each method? (TC1A-Table 8.3.4) |  |  |  |  |
|  | - Do the specification exams address the equipment, operating procedure, and NDT techniques that the individual may encounter during specific assignments? (TC1A- 8.4.1) |  |  |  |  |
|  | - Are there practical (operational) exams for Level I and Level II for each method? (TC1A- 8.5) |  |  |  |  |
|  | - Is at least one flawed specimen or component used to test personnel? (TC1A- 8.5.2) |  |  |  |  |
|  | - For Film Interpretation certification, does the practical exam consist of at least 20 images (radiographs)? (TC1A- 8.5.2.2) |  |  |  |  |
|  | - Are there checklists (attributes) with at least 10 different checkpoints for each method? (TC1A- 8.5.4) |  |  |  |  |
|  | - Are there critical (pass/fail) checkpoints on the checklist? (TC1A- 8.5.5) |  |  |  |  |
|  | - Are all exam based upon a composite score of 80% with no individual score of less than 70%? (TC1A-8.1.5) |  |  |  |  |
|  | - Does a “qualified representative” of the Level III Examiner administer written exams? Is there documented evidence of this qualification and appropriate training by the Level III? (TC1A-8.1.2) |  |  |  |  |
|  | - Does a “qualified representative” of the Level III Examiner administer practical exams? Are they certified in the applicable method as a Level II or III? (TC1A -8.1.2) |  |  |  |  |
|  | - Is there evidence the “Examiner Specific Exam” has been administered to the Level III Examiner? (TP271- 1.6.2.1) |  |  |  |  |
|  | - Is there evidence the Level III Examiner has taken the “basic” exam prior to their initial certification? (TC1A- 8.7.1) |  |  |  |  |
|  | - Are certification exams being administered by one’s self or a subordinate (this is not allowed)? (TC1A- 8.1.8) |  |  |  |  |
| e | NSTR-99 |  |  |  |  |
|  | Written Exams |  |  |  |  |
|  | - Are there written exams for each inspection method performed at the supplier? |  |  |  |  |
|  | - Do operator only (Level I) written exams contain at least 25 questions and do they contain the required mix? (NSTR99- 5.0.B.1, Table 1) |  |  |  |  |
|  | - Do Inspector (Level II) written exams contain at least 50 questions and do they contain the required mix? (NSTR99- 5.0.B.1, Table 1) |  |  |  |  |
|  | - Are two answer questions limited to 20% of the exam? (NSTR99- 5.0.B.2) |  |  |  |  |
|  | - Are there at least 2 separate exams for each method or if an exam generator is used are there enough questions for at least two different exams? (NSTR99- 5.0.A.6) |  |  |  |  |
|  | - Do exams have maximum of 20% repeat questions? (NSTR99- 5.0.A.7) |  |  |  |  |
|  | - Do written exams have uniform point values? (NSTR99- 5.0.A.8) |  |  |  |  |
| f | Practical Exams |  |  |  |  |
|  | - Are there practical exams for each inspection method performed at the supplier? |  |  |  |  |
|  | - Do practical exams contain artificial or natural flaws and are grossly rejectable flaws avoided? (NSTR99-5.0.C.1) |  |  |  |  |
|  | - Are specific point values assigned to each indication and important aspects of the technique? (NSTR99- 5.0.C.3) |  |  |  |  |
|  | - Are specification exam questions current to the most recent revision of the applicable specifications? |  |  |  |  |
|  | - Are test props mapped, photographed or otherwise recorded? (NSTR99- 5.0.C.3) |  |  |  |  |
|  | - Are key portions of the practical exam monitored by the Examiner or a delegated representative? (NSTR99- 5.0.A.12) |  |  |  |  |
| 5 | Vision Test Requirements |  |  |  |  |
| a | - Is there a J-1 vision exam record for each Level I, II and III? (TP271, 1.6.7, MS2132, 10.7, NS1500, 6.7.3) |  |  |  |  |
| b | - Is there a color discrimination vison record for each Level I, II and III? (TP271, 1.6.7, MS2132, 10.7, NS1500, 6.7.3) |  |  |  |  |
| c | - For individuals with impaired color vision (color blindness) is there a record of an alternative color perception exam for discerning colors of the different examination methods? (TP271, 1.6.7, MS2132, 10.7, NS1500, 6.7.3) |  |  |  |  |
| 6 | Alternate Qualification Program (NSTR-99) |  |  |  |  |
| a | - Does the supplier use an alternate qualification program? (NSTR99- 8.0) |  |  |  |  |
| b | - If the supplier applies the alternate program, does the supplier apply NSTR-99 paragraphs 2.0, 3.0, 6.0 and 7.0 (this is required)? (NSTR99- 8.0) |  |  |  |  |
| c | - If the alternate program uses the SNT-TC-1A, see attributes above, (SNT-TC-1A requirements are mandatory). (NSTR99, 8.0) |  |  |  |  |
| d | - Is the supplemental written exam with a minimum of 20 questions related to NNPP administered to Level II personnel? (NSTR99- 8.0.d) |  |  |  |  |
| 7 | Other Concerns or issues |  |  |  |  |

| Overall MPS Results: | SATISFACTORY |  | UNSATISFACTORY |  |
| --- | --- | --- | --- | --- |

| Corrective Action Generated? | No |  |  | Yes |  |  | CAR# |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

FOLLOW-UP ACTION REQUIRED?

SUMMARY/NOTES/COMMENTS/CONCERNS: